| FRONT SUSPENSION |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Shock Absorber Upper Nut | 27 | 20 | — |
| Shock Absorber Lower Nut | 76 | 56 | — |
| Suspension Arm Lower Axle Bracket Nut | 169 | 125 | — |
| Suspension Arm Lower Frame Bracket Nut | 169 | 125 | — |
| Suspension Arm Upper Axle Bracket Nut/Bolt | 102 | 75 | — |
| Suspension Arm Upper Frame Bracket Bolt | 102 | 75 | — |
| Stabilizer Bar Retainer Bolts | 61 | 45 | — |
| Stabilizer Bar Link Upper Nut | 102 | 75 | — |
| Stabilizer Bar Link Lower Bolt | 102 | 75 | — |
| Track Bar Frame Bracket Nut | 169 | 125 | — |
| Track Bar Axle Bracket Bolt | 169 | 125 | — |
| Hub/Bearing Bolts | 102 | 75 | — |
| Hub/Bearing Axle Nut | 136 | 100 | — |
| Jounce Cup Bolt | 42 | 31 | — |
| Drag link to Knuckle (Right Side) | 74 | 55 | — |
| Drag link to Knuckle | 74 | 55 | — |
| Axle Shaft Nut | 136 | 100 | — |
| Wheel speed sensor | 4-6 | — | 34-50 |

| REAR SUSPENSION |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Shock Absorber Upper Bolts | 50 | 37 | — |
| Shock Absorber Lower Nut | 76 | 56 | — |
| Suspension Arm Lower Axle Bracket Nut | 169 | 125 | — |
| Suspension Arm Lower Frame Bracket Bolt | 169 | 125 | — |
| Suspension Arm Upper Axle Bracket Bolt | 169 | 125 | — |
| Suspension Arm Upper Frame Bracket Bolt | 169 | 125 | — |
| Stabilizer Bar Retainer Bolts | 61 | 45 | — |
| Stabilizer Bar Link Nut/Bolt | 102 | 75 | — |
| Stabilizer Bar to Link Nut | 90 | 66 | — |
| Track Bar Frame Bracket Nut | 169 | 125 | — |
| Track Bar Axle Bracket Bolt | 169 | 125 | — |

| PROPELLER SHAFT |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Front Shaft - Axle Flange Bolts | 110 | 81 | - |
| Front Shaft - Transfer Case Flange Bolts | 20 | 15 | - |
| Rear Shaft - Axle Flange Bolts | 20 | 15 | - |
| Rear Shaft - Transfer Case Flange Bolts | 20 | 15 | - |

| FRONT AXLE - 186FBI |  |
| --- | --- |
| DESCRIPTION | SPECIFICATION |
| Axle Ratio | 3.21, 4.10 |
| Ring Gear Backlash | 0.12 - 0.20 mm (0.005 - 0.008 in.) |
| Pinion Torque To Rotate - Original Bearings | 1.13 - 2.26 N·m (10 - 20 in. lbs.) |
| Pinion Torque To Rotate - New Bearings | 2 - 3.4 N·m (15 - 30 in. lbs.) |
| Differential Total Torque To Rotate: Pinion Torque To Rotate Plus | 0.79 - 1.24 N·m (7 - 11 in. lbs.) |

| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| --- | --- | --- | --- |
| Fill Hole Plug | 34 | 25 | - |
| Differential Cover Bolts | 41 | 30 | - |
| Bearing Cap Bolts | 61 | 45 | - |
| Ring Gear Bolts | 108 | 80 | - |
| Pinion Bearing Nut Min - Max | 217 - 542 | 160 - 400 | - |
| Axle Nut | 136 | 100 | - |
| Hub Bearing Bolts | 102 | 75 | - |

| FRONT AXLE - 216FBI |  |
| --- | --- |
| DESCRIPTION | SPECIFICATION |
| Axle Ratio | 4.1 |
| Ring Gear Backlash | 0.12 - 0.20 mm (0.005 - 0.008 in.) |
| Pinion Torque To Rotate - Original Bearing | 1 - 2 N·m (10 - 20 in. lbs.) |
| Pinion Torque To Rotate - New Bearing | 2 - 4.5 N·m (20 - 40 in. lbs.) |
| Total Torque To Rotate: Pinion Torque To Rotate plus | 0.79 - 1.24 N·m (7 - 11 in lbs) |

| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| --- | --- | --- | --- |
| Fill Plug | 34 | 25 | - |
| Differential Cover Bolts | 41 | 30 | - |
| Bearing Cap Bolts | 108 | 80 | - |
| Pinion Nut | 217-271 | 160-200 | - |
| Ring Gear Bolts | 136 | 100 | - |
| Axle Nut | 136 | 100 | - |
| Hub Bearing Bolts | 102 | 75 | - |

| REAR AXLE - 198RBI |  |
| --- | --- |
| DESCRIPTION | SPECIFICATION |
| Axle Ratio | 4.1 |
| Ring Gear Backlash | 0.12 - 0.20 mm (0.005 - 0.008 in.) |
| Pinion Torque To Rotate - Original Bearings | 1 - 2 N·m (10 - 20 in. lbs.) |
| Pinion Torque To Rotate - New Bearings | 2 - 4 N·m (20 - 35 in. lbs.) |
| Total Torque To Rotate: Pinion Torque To Rotate plus | 0.79 - 1.24 N·m (7 - 11 in. lbs.) |

| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| --- | --- | --- | --- |
| Differential Cover Bolts | 41 | 30 | - |
| Bearing Cap Bolts | 77 | 57 | - |
| Ring Gear Bolts | 136 | 100 | - |
| Pinion Nut | 271 - 475 | 200 - 350 | - |
| Axle Retainer Nuts | 61 | 45 | - |

| REAR AXLE - 226RBI |  |
| --- | --- |
| DESCRIPTION | SPECIFICATION |
| Axle Ratio | 3.21, 4.10 |
| Ring Gear Backlash | 0.12 - 0.20 mm (0.005 - 0.008 in.) |
| Pinion Torque To Rotate - Original Bearings | 1 - 2 N·m (10 - 20 in. lbs.) |
| Pinion Torque To Rotate - New Bearings | 2 - 4.5 N·m (20 - 40 in. lbs.) |
| Total Torque To Rotate: Pinion Torque To Rotate plus | 0.79 - 1.24 N·m (7 - 11 in. lbs.) |

| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| --- | --- | --- | --- |
| Fill Hole Plug | 34 | 25 | - |
| Differential Cover Bolts | 41 | 30 | - |
| Bearing Cap Bolts | 108 | 80 | - |
| Ring Gear Bolts | 136 | 100 | - |
| Pinion Nut | 217 - 271 | 160 - 200 | - |
| Axle Retainer Nuts | 61 | 45 | - |

| BRAKE COMPONENTS |  |
| --- | --- |
| DESCRIPTION | SPECIFICATION |
| Disc Brake Caliper Type | Sliding |
| Front Disc Brake Rotor Type | Ventilated |
| Rear Disc Brake Rotor Type | Solid |
| Front Disc Brake Rotor Diameter | 302 x 28 mm (11.89 x 1.10 in.) |
| Rear Disc Brake Rotor Diameter | 312 x 12 mm (12.28 x 0.472 in.) |
| Front Ventilated Disc Brake Rotor | Max. Runout 0.10 mm (0.0039 in.) |
| Rear Solid Disc Brake Rotor | Max. Runout 0.102 mm (0.004 in) |
| Front Ventilated Disc Brake Rotor | Max. Thickness Variation 0.009 mm (0.00035 in.) |
| Rear Solid Disc Brake Rotor | Max Thickness Variation 0.018 mm (0.0007 in) |
| Front Ventilated Disc Brake Rotor | Min. Thickness 22.7 mm (0.8937 in.) |
| Rear Solid Disc Brake Rotor | Min. Thickness 11.00 mm (0.433 in.) |
| Brake Booster Type | Tandem Diaphragm |

| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| --- | --- | --- | --- |
| Brake Booster Mounting Nuts | 25 | 18 | — |
| Master Cylinder Mounting Nuts | 17 | 13 | 151 |
| Booster Mounting Plate | 18 | — | 159 |
| Master Cylinder Brake Lines | 19 | 14 | 168 |
| Front Caliper Adapter Mounting Bolts | 35 | 26 | — |
| Rear Caliper Adapter Mounting Bolts | 35 | 26 | 220 |
| Front Caliper Brake Hose Banjo Bolt | 31 | 23 | — |
| Rear Caliper Brake Hose Banjo Bolt | 31 | 23 | — |
| Front Caliper adapter | 163 | 120 | — |
| Rear Caliper adapter | 75 | 55 | — |
| Parking Brake Lever Bracket Bolts | 12 | 9 | — |

| CLUTCH |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Pressure Plate Bolts - 2.8L | 31 | 23 | - |
| Pressure Plate Bolts - 3.8L | 50 | 37 | - |
| Flywheel Bolts - 2.8L | 95 | 70 | - |
| Flywheel Bolts - 3.8L | 81 | 60 | - |
| Slave Cylinder Nuts | 23 | 17 | - |
| Dust Shield Bolts | 50 | 37 | - |
| Crossmember Frame Bolts | 41 | 30 | - |

| COOLING SYSTEM |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Bolt, Block Heater | 2 | - | 17 |
| Bolts, Condenser to Radiator |  |  |  |
| Electric Fan to Fan Shroud bolts | 6 |  | 53 |
| Bolts, Fan Blade Assembly to Viscous Drive - Diesel | 23 | - | 210 |
| Viscous Fan Drive - Diesel | 50 | 37 | - |
| Bolts, Fan Shroud to Radiator Mounting | 8 | - | 70 |
| Bolts, Radiator Upper Isolator to Crossmember | 9 | - | 80 |
| Bolts, Thermostat Housing- | 28 | - | 250 |
| Bolts, Water Pump |  |  |  |
| 3.8L | 12 | - | 105 |
| 2.8L | 23 | - | 200 |

| 3.8L ENGINE |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Engine Type | 60° V-6 Engine |  |
| Number of Cylinders | 6 |  |
| Displacement 3.8L | 3.8 Liters | 231 cu. in. |
| Bore 3.8L | 96.0 mm | 3.779 cu.in. |
| Stroke | 87 mm | 3.425 in. |
| Compression Ratio 3.8L | - | 9.6:1 |
| Firing Order | - | 1-2-3-4-5-6 |
| Compression Pressure-Minimum | 689.5 kPa | 100 psi. |
| Cylinder Compression (Max. Difference Between Cylinders | - | 0.25 |

| CYLINDER BLOCK |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Cylinder Bore Diameter (Standard) 3.8L | 95.993-96.007 mm | 3.7792-3.780 in. |
| Out of Round (Service Limits) | 0.076 mm | 0.003 in. |
| Taper (Service Limits) | 0.051 mm | 0.002 in. |
| Lifter Bore Diameter | 22.980-23.010 mm | 0.905-0.906 in. |
| Deck Surface Flatness (Max.) | 0.1 mm | 0.004 in. |

| CRANKSHAFT |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Connecting Rod Journal Diameter | 57.979-58.005 mm | 2.2827-2.2837 in. |
| Main Bearing Journal Diameter | 63.993-64.013 mm | 2.5194-2.5202 in. |
| Journal Out-of-Round (Max.) | 0.025 mm | 0.001 in. |
| Journal Taper (Max.) | 0.025 mm | 0.001 in |
| End Play | 0.09-0.24 mm | 0.0036-0.0095 in. |
| Wear Limit | 0.381 mm | 0.015 in. |
| Main Bearing Diametrical Clearance 1-2-3-4 | 0.011-0.055 mm | 0.0005-0.0022 in. |
| Wear Limit | 0.076 mm | 0.003 in. |

| CONNECTING RODS |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Bearing Clearance | 0.019-0.065 mm. | 0.017-0.020 in. |
| Wear Limit | 0.074 mm | 0.003 in. |
| Side Clearance | 0.13-0.32 mm | 0.005-0.013 in. |
| Wear Limit | 0.38 mm | 0.015 in. |

| PISTONS |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Piston Diameter 3.8L-Measured 33.01 mm (1.30 in) From Piston Top | 95.968-95.998 mm. | 3.778-3.779 in. |
| Clearance in Bore @ Size Location (New) | -0.005-0.039 mm | -0.0002–0.0015 in. |
| Weight 3.8L | 426 ± 5 grams | 15.03 ± 0.1764 oz. |

| PISTON PINS |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Type | Press Fit in Rod (Serviced as a Assembly) |  |
| Clearance in Piston @ 21C (70°F) | 0.006-0.019 mm | 0.0002-0.0007 in. |
| Clearance in Connecting Rod | Interference Fit |  |
| Diameter | 22.87-22.88 mm | 0.9007-0.9009 in. |
| Length 3.8L | 71.25-71.75 mm | 2.805-2.824 in. |

| PISTON RINGS |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Ring End Gap |  |  |
| Top Compression Ring | 0.18-0.38 mm | 0.007-0.015 in. |
| Second Compression Ring | 0.28-0.57 mm | 0.011-0.022 in. |
| Oil Control (Steel Rails) | 0.23-0.78 mm | 0.009-0.030 in. |
| Wear Limit-Compression Rings | 1.0 mm | 0.039 in. |
| Wear Limit-Oil Control Steel Rails | 1.88 mm | 0.074 in. |
| Ring Side Clearance |  |  |
| Top Compression Ring 3.8L | 0.030-0.069 mm | 0.0012-0.0027 in. |
| Second Compression Ring 3.8L | 0.041-0.085 mm | 0.0016-0.0033 in. |
| Oil Ring (Steel Ring) | 0.039-0.200 mm | 0.0015-0.0078 in. |
| Wear Limit- Top Ring | 0.10 mm | 0.004 in. |
| Wear Limit-2nd Ring | 0.13 mm | 0.005 |
| Wear Limit Oil Ring Pack | 0.266 mm | 0.009 |
| Ring Width-Top Compression Ring 3.8L | 1.175-1.190 mm | 0.0462-0.0468 |
| Ring Width-2nd Compression Ring 3.8L | 1.46-1.49 mm | 0.0575-0.058 in. |
| Ring Width-Oil Ring (Steel Rails) 3.8L | 0.435-0.510 mm | 0.017-0.020 |

| CAMSHAFT |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Journal Diameter |  |  |
| #1 | 50.724-50.775 mm | 1.997-1.999 in. |
| #2 | 50.317-50.368 mm | 1.9809-1.9829 in. |
| #3 | 49.936-49.987 mm | 1.9659-1.9679 in. |
| #4 | 49.530-49.581 mm | 1.9499-1.9520 in. |
| Bearing Clearance-Diametrical | 0.025-0.101 mm | 0.001-0.004 in. |
| Bearing Clearance (Max.Allowable) | 0.127 mm | 0.005 in. |
| End Play | 0.254-0.508 mm | 0.010-0.020 in. |
| Camshaft Bearing Diameter |  |  |
| #1 | 50.800-50.825 | 1.9999-2.0009 in. |
| #2 | 50.393-50.419 mm | 1.9839-1.9849 in. |
| #3 | 50.013-50.038 mm | 1.9690-1.9699 in. |
| #4 | 49.606-49.632 mm | 1.9529-1.954 in. |
| Exhaust Valve Timing |  |  |
| Closes-3.8L (ATDC) | - | 18° |
| Opens-3.8L (BBDC) | - | 46° |
| Duration-3.8L | - | 244° |
| Intake Valve Timing |  |  |
| Closes-3.8L (ABDC) | - | 63° |
| Opens-3.8L (ATDC) | - | 1° |
| Duration-3.8L | - | 242° |
| Valve Overlap-3.8L | - | 17° |

| HYDRAULIC LIFTER |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Type | Hydraulic Roller |  |
| Outside Diameter | 22.949-22.962 mm | 0.903-0.904 in. |
| Clearance in Block | 0.020-0.061 mm | 0.0007-0.0024 in. |

| CYLINDER HEAD |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Gasket Thickness (Compressed) | 0.65-0.75 mm | 0.0007-0.0024 in. |

| VALVES |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Face Angle-Intake | - | 45-45.5° |
| Face Angle-Exhaust | - | 45-45.5° |
| Head Diameter-Intake | 47.87-48.13 mm | 1.88-1.89 in. |
| Head Diameter-Exhaust | 35.37-35.63 mm | 1.39-1.40 in. |
| Valve Lift (Zero Lash)-Intake and Exhaust-3.8L | 11.0 mm | 0.433 in. |
| Valve Length-Intake | 125.84-126.6 mm | 4.95-4.98 in. |
| Valve Length-Exhaust | 127.20-127.96 | 5.00-5.04 in. |
| Valve Stem to Tip Height (valve tip to spring seat washer)-Intake | 48.1-49.7 mm | 1.89-1.95 in. |
| Valve Stem to Tip Height (valve tip to spring seat washer)-Exhaust | 48.53-50.09 mm | 1.91-1.97 in. |

| VALVE SEAT |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Angle | - | 44.5-45° |
| Run Out (Service Limits) | 0.0762 mm | 0.003 in. |
| Width-Intake and Exhaust | 1.50-2.00 mm | 0.057-0.078 in. |

| VALVE GUIDE |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Guide Bore Diameter (Std.) | 6.975-7.00 mm | 0.274-0.275 in. |

| VALVE MARGIN |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Intake | 0.825-0.973 mm | 0.032-0.038 in. |
| Exhaust | 1.565-1.713 mm | 0.061-0.067 in. |

| VALVE STEM DIAMETER |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Intake (Standard) | 6.935-6.953 mm | 0.2718-0.2725 in. |
| Exhaust (Standard) | 6.906-6.924 mm | 0.2718-0.2725 in. |

| VALVE STEM TO GUIDE CLEARANCE |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Intake | 0.025-0.065 mm | 0.001-0.0025 in. |
| Exhaust | 0.059-0.094 mm | 0.002-0.0037 in. |
| Max Allowable-Intake (Rocking Method) | 0.247 mm | 0.010 in. |
| Max Allowable-Exhaust (Rocking Method) | 0.414 mm | 0.016 in. |

| PUSH RODS |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Length | 135.438 mm | 5.33 in. |

| VALVE SPRING |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Free Length-Type A | 51.4 mm | 2.02 in. |
| Free Length-Type B | 53.4 mm | 2.10 in. |
| Wire Diameter Type A | 3.95-4.77 mm | 0.15-0.19 in. |
| Wire Diameter Type B | 4.19-4.29 mm | 0.16-0.17 in. |
| Number of Coils Type A | 7.52 |  |
| Number of Coils Type B | 7.25 |  |
| Spring Tension (Valve Closed) Type A | 376.4-424.4 N @ 41.9 mm | 84.6-95.6 lbs. @ 1.65 in. |
| Spring Tension (Valve Open) Type A | 863.9-959.9 N @ 41.9 mm | 194.2-215.8 lbs. @ 1.65 in. |
| Spring Tension (Valve Closed) Type B | 377-423 N @ 41.9 mm | 84.8-95.2 lbs. @ 1.65 in. |
| Spring Tension (Valve Open) Type B | 880-962 N @ 30.91 mm | 197.9-216.3 lbs. @ 122 in. |
| Installed Height | 41.1-42.7 mm | 1.61-1.68 in |

| LUBRICATION |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| At Curb Idle Speed* (Minimum with engine at operating temperature) | 34.47 kPa | 5 psi |
| At 3000 RPM | 205-551 kPa | 30-80 psi |
| Oil Filter By-Pass Valve Setting | 62-103 kPa | 9-15 psi |
| Oil Pressure Switch Actuating Pressure | 14-28 Kpa | 2-4 psi |
| CAUTION: *If pressure is ZERO at curb idle, DO NOT run engine at 3000 rpm. |  |  |

| OIL PUMP |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Clearance Over rotors-Inner and Outer | 0.10 mm | 0.004 in. |
| Cover Out-Of-Flat (Max.) | 0.025 mm | 0.001 in. |
| Inner Rotor Thickness | 7.64 mm | 0.301 in. |
| Outer Rotor Thickness (Min) | 7.64 mm | 0.301 in. |
| Outer Rotor Clearance (Max) | 0.039 mm | 0.015 in. |
| Outer Rotor Diameter (Min) | 79.95 mm | 3.148 in. |
| Tip Clearance Between Rotors (Max) | 0.20 mm | 0.008 in. |

| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| --- | --- | --- | --- |
| Camshaft Sprocket—Bolt | 54 | 40 | — |
| Camshaft Thrust Plate—Bolts | 12 | — | 105 |
| Forged Connecting Rod Cap-Nuts | 54 +¼ turn | 40 +¼ turn | — |
| Cracked Connecting Rod Cap-Bolts |  |  |  |
| Crankshaft Main Bearing Cap-Bolts | 41 +¼ turn | 30 +¼ turn | — |
| Crankshaft Main Bearing Cap Cross Bolts (3.8L) | 61 | 45 | — |
| Crankshaft Oil Seal Retainer Rear-Bolts | 12 | — | 105 |
| Crankshaft Damper-Bolt | 54 | 40 | — |
| Cylinder Block Drain Plugs | 20 | 15 | — |
| Cylinder Head-Bolts |  |  |  |
| Cylinder Head Cover-Bolts | 12 | — | 105 |
| Flex Plate to Crankshaft | 88 | 65 | — |
| Engine Mounting |  |  |  |
| Exhaust Manifold-Bolts | 23 | — | 200 |
| Intake Manifold - Lower-Bolts | 23 | — | 200 |
| Intake Manifold - Lower Gasket Retainer-Bolts | 12 | — | 105 |
| Intake Manifold Upper-Bolts | 12 | — | 105 |
| Lifter Yoke Retainer-Bolts | 12 | — | 105 |
| Oil Filter Attaching Fitting | 54 | 40 | — |
| Oil Cooler Attaching Fitting | 27 | 20 | — |
| Oil Filter | 20 | 15 | — |
| Oil Filter Adapter-Bolts | 28 | — | 250 |
| Oil Gallery Plug | 27 | 20 | — |
| Oil Pan—Bolts | 12 | — | 105 |
| Oil Pan Drain-Plug | 27 | 20 | — |
| Oil Pressure Switch | 23 | — | 200 |
| Oil Pump Cover Plate-Screws | 12 | — | 105 |
| Oil Pump Pick-up Tube-Bolt | 28 | — | 250 |
| Oil Dipstick Housing-Bolts | 48 | 35 | — |
| Rocker Arm Shaft-Bolts | 23 | — | 200 |
| Spark Plug | 16 | 12 | — |
| Thermostat Housing | 12 | — | 105 |
| Timing Chain Case Cover |  |  |  |
| —M8 Bolt | 27 | 20 | — |
| —M10 Bolt | 54 | 40 | — |
| Water Pump-Bolts | 12 | — | 105 |
| Water Pump Pulley-Bolts | 28 | — | 250 |

| 2.8L Panther Engine |  |
| --- | --- |
| Engine | 2.8L PANTHER JK |
| Engine Type | 2.8L — 16 Valves |
| Displacement | 2777 cc |
| Bore | 94 |
| Stroke | 100.05 |
| Power (VGT) | 130 kW (177CV)@3800 RPM |
| Torque (ATX) | 400 Nm @ 2000 RPM |
| Torque (MTX) | 410 Nm @ 2000 RPM |
| Cylinders | 4 In line |
| Injection Order | 1–3–4–2 |
| Compression Ratio | 17.0:1 |
| Maximum compression pressure variation between two cylinders. | 5 bar |
| Vacuum at idle | 680 mm/HG (27.5 In/HG) |
| Idle Speed (ATX) | 760 +/- 50 RPM |
| Idle Speed (MTX) | 875 +/- 50 RPM |
| Maximum RPM in Gear | 4500 RPM |
| Maximum RPM in neutral | ATX 2800 MTX 3500 |
| Belt tension | Automatic Belt Tensioner |
| Thermostat opening | 80°C +/- 2°C |
| Generator Rating | Denso 12V-180A |
| Emissions Level | EU4 |
| Block configuration/Material | Open/Cast Iron |
| Cylinder Head | Dual Overhead Cam |
| Timing System | Belt |
| Fuel System | CP3.2+ 1,600 bar Fuel Pump, Piezo Injectors |
| Electronic Control Unit | EDC 16 |
| Timing System | Belt Driven DOHC Overhead Camshaft |
| Air Intake | Dry Filter With turbocharger and Charge Air Cooler |
| Fuel System | Direct Fuel Injection Common Rail System |
| Emission devices | Cooled EGR (pneumatic) |
|  | Electric Intake Throttle |
|  | Fast Metallic Glow plugs |
| Combustion Cycle | 4 Stroke |
| Cylinder Compression Difference Between Cylinders | 5 bar (72.5 psi) |
| Cooling System | Water Cooling |
| Turbocharging: | Single VGT with REA |
| Intake Ports | Aluminum heads with traditional dual side intake and exhaust ports. One intake port is helical and the other has a directed entry. |
| Crankshaft | 8 Counterweights with an incorporated balance shaft gear. |
| Camshafts | 2 overhead camshafts with axial front bearings and identical camshaft caps, finger followers, and hydraulic lifters. |
| Intake & Exhaust Valves | Flat with fire deck face. |
| Intake Manifold | Aluminum, with Cast-in EGR passages, intake mixer, vacuum actuated EGR valve, electric intake throttle and a U-type EGR cooler |
| Lubrication | Pressure Lubricated By Rotary Pump |
| Engine Rotation | Clockwise Viewed From Front Cover |

| 2.8L Panther Engine Specifications |  |
| --- | --- |
| Cylinder Head |  |
| Cylinder head height | 135.5 mm (5.334 in.) |
| Cylinder head flatness deformation tolerance | 0.075 mm (0.003 in.) |
| Cylinder head gasket thickness |  |
| 0 Hole | 1.10 mm ( 0.043 in) |
| 1 Hole | 1.20 mm (0.047 in) |
| 2 Holes | 1.30 mm (0.051 in) |
| Intake Manifold |  |
| Intake manifold flatness deformation tolerance | 0.075 mm (0.003 in.) |
| Exhaust Manifold |  |
| Exhaust manifold flatness deformation tolerance | 0.075 mm (0.003 in.) |
| Tappets |  |
| Hydraulic tappet outside diameters | 11.994 mm +/- 0.06 mm (0.472 in +/- 0.002) |
| Valves |  |
| Intake valve face angle | 45°30' |
| Exhaust valve face angle | 45°30' |
| Intake Valve Head Diameter | 32 mm (1.25 in.) |
| Exhaust Valve Head Diameter | 29.4 mm (1.15 in.) |
| Intake Valve Stem Diameter | 5.97 mm (0.235 in.) |
| Exhaust Valve Stem Diameter | 5.96 mm (0.235 in.) |
| Intake Valve Guide Stem Clearance |  |
| Min | 0.030 mm (0.0012 in.) |
| Max | 0.060 mm (0.0024 in.) |
| Exhaust Valve Guide Stem Clearance |  |
| Min | 0.040 mm (0.0016 in.) |
| Max | 0.070 mm (0.0028 in.) |
| Valve Springs |  |
| Free Length | 50.8 mm (2 in.) |
| Closed Valve | 38 mm (1.49 in.) |
| Opened Valve | 29 mm (1.14 in.) |
| Camshafts |  |
| Camshaft End Play |  |
| Max | 0.350 mm (0.013 in.) |
| Min | 0.150 mm (0.006 in.) |
| Outer Journal Diameter (at crankshaft) | 25.95 mm +/– 0.01 mm (1.021 mm +/- .0004 in) |
| Inner Journal Diameter (at cylinder head) | 26.00 mm +/– 0.015 mm (1.027 mm +/- .0006 in) |
| Crankshaft Journal Clearance. |  |
| Max | 0.075 mm (0.003 in.) |
| Min | 0.030 mm (0.0012 in) |
| Connecting Rods |  |
| Connecting Rod Diameter (Small End) | 32 mm (1.26 in.) |
| Connecting Rod Diameter (Large End) | 57.563 mm ( 2.266 in.) |
| Piston Pin |  |
| Diameter | 32 mm (1.26 in.) |
| Length | 70.7 mm - 71.00 mm (2.78 in — 2.79 in.) |
| Crankshaft |  |
| End Play | 0.1 mm — 0.33 mm (0.004 in. — 0.013 in.) |
| Bearing Selection |  |
| Engine Block |  |
| Cylinder Bore Internal Diameter | 94 mm (3.700 in.) |
| Cylinder Bore Taper | TBD |
| Cylinder Bore Out-Of-Round | 0.007 mm (0.0003 in.) |
| Fuel System |  |
| Injection Pressure | CRS 3.0 — 1600 Bar |
| High Pressure Pump | CP3.2+ |
| ECU | EDC16CP31 |
| Injectors | Piezo CRI 3.0 |
| Glow Plugs |  |
| Make/Type | Bosch / GLP2 |
| Voltage | 4.4V |
| Lubrication System |  |
| Oil Pump Outer Rotor End Play |  |
| Min | 0.01 (0.0004 in.) |
| Max | 0.09 (0.0036 in.) |
| Oil Pump Inner Rotor End Play |  |
| Max | 0.01 mm (0.0004 in.) |
| Min | 0.09 mm (0.0036 in.) |
| Oil Pump Outer Rotor to Body Diameter Clearance |  |
| Max | 0.130 mm (0.052 in.) |
| Min | 0.230 mm (0.0091 in) |
| Oil Pressure Relief Valve |  |
| Opening Pressure | 5 Bar |
| Oil Pressure Valve Spring Free Length | 46.8 mm (1.84 in) |
| Minimum Oil Pressure (Warm) |  |
| at Idle | 0.7 Bar |
| at 3800 RPM | 2.5 Bar |
| Cooling System |  |
| Thermostat Opening Temperature | 80°C (176°F) |
| Engine Oil |  |
| Coolant |  |

| ENGINE BLOCK |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Lower Oil Pan | 15 | 12 | 133 |
| Upper Oil Pan (m6 bolt) | 15 | 12 | 133 |
| Upper Oil Pan (m8 bolt) | 32 | 23 | 283 |
| Oil Pickup Tube | 15 | 12 | 133 |
| Oil Drain Plug | 54 | 40 | 478 |
| Balance Shaft | 32 | 23 | 283 |
| Main Bearing Caps |  |  |  |
| Connecting Rod Caps |  |  |  |
| Dipstick Tube (sump) | 11 | 8 | 97 |
| Dipstick Tube (block) | 33 | 24 | 292 |
| Oil Cooler Coolant Line | 15 | 12 | 133 |
| Transmission adapter bolts (hex head) | 15 | 12 | 133 |
| Transmission adapter bolts (Allen Head) | 78 | 57 | 690 |
| Oil Jet | 11 | 8 | 97 |
| Engine Mount Bolts | 12 | 9 | 106 |
| Oil Filter Cap | 25 | 18.4 | 221 |
| Oil Pressure Sensor | 14 | 10 | 124 |
| Air Temp/Pressure sensor | 12 | 9 | 106 |

| CYLINDER HEAD |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Cylinder Head Bolt |  |  |  |
| Cylinder Head Cover | 11 | 8 | 97 |
| Vacuum Tube | 11 | 8 | 97 |
| Fuel Rail | 24 | 18 | 18 |
| Fuel Injector Lines at the injector | 28 | 20 | 247 |
| Fuel injector lines at the fuel rail | 20 | 15 | 177 |
| High Pressure Fuel Feed Line at the Fuel Rail | 20 | 15 | 177 |
| High Pressure Fuel Feed Line at the High Pressure Pump | 28 | 20 | 247 |
| High Pressure Fuel Line Bracket bolt | 15 | 11 | 133 |
| Fuel injector | 33 | 24 | 292 |
| Glow Plugs | 14 | 10 | 124 |
| EGR Throttle Assembly | 11 | 8 | 97 |
| Exhaust Manifold | 35 | 29 | 309 |
| Camshaft Sprocket | 64 | 47 | 566 |
| Camshaft Cap | 11 | 8 | 97 |
| Front Camshaft Journal | 11 | 8 | 97 |
| Intake Manifold | 25 | 19 | 221 |

| FRONT ENGINE |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Outer Front Cover (upper) | 11 | 8 | 97 |
| Outer Front Cover (lower) | 11 | 8 | 97 |
| Inner Front Cover | 11 | 8 | 97 |
| Front Cover | 15 | 12 | 133 |
| Crankshaft Sprocket | 100 + 75° | 74+ 75° | 885+ 75° |
| Crankshaft Pulley | 32 | 23 | 283 |
| Front Engine Lifting Bracket | 45 | 33 | 398 |
| Accessory Drive Idler Pulley | 45 | 33 | 398 |
| CMP sensor | 11 | 8 | 97 |
| Water Pump | 32 | 23 | 283 |
| Timing Belt Tensioner | 20 | 15 | 177 |

| REAR ENGINE |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Rear Cover | 15 | 10 | 133 |
| Rear Lifting Bracket | 45 | 33 | 398 |
| Crankshaft Sensor | 11 | 8 | 97 |
| CKP Cover Plate | 15 | 11 | 133 |
| Trans Adapter Plate (allen bolts) | 79 | 58 | 699 |
| Trans Adapter Plate (hex bolt) | 45 | 33 | 398 |

| ACCESSORY DRIVE |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Power Steering Pump Pulley | 33 | 24 | 283 |
| Power Steering Pump | 33 | 24 | 283 |
| Generator | 33 | 24 | 283 |
| Generator Bracket | 45 | 33 | 398 |
| A/C Compressor | 32 | 23 | 283 |
| A/C Compressor Bracket | 45 | 33 | 33 |
| Turbocharger | 32 | 23 | 283 |
| Turbocharger Oil Feed Line at the Turbocharger | 24 | 18 | 212 |
| Turbocharger Oil Feed Line at the Engine Block | 32 | 23 | 283 |
| Turbocharger Oil Return Line | 15 | 11 | 133 |
| Turbocharger Brace | 32 | 23 | 283 |
| Turbocharger Adapter (oil feed line to engine block connection) | 54 | 40 | 478 |
| EGR Valve | 15 | 11 | 133 |
| EGR Cooler | 15 | 11 | 133 |
| High Pressure Fuel Pump | 24 | 18 | 212 |
| High Pressure Fuel Pump Sprocket. | 88 | 65 | 779 |
| Oil Cooler Housing | 32 | 23 | 283 |
| Oil Cooler Feed Line | 11 | 8 | 97 |

| EXHAUST |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Bolts, Crossmember to Sill | 42 | 31 | - |
| Nuts, Crossmember to Transmission Mount | 22 | 16 | - |
| Nuts/Bolt, Exhaust Manifold to Engine |  |  |  |
| #6&7 | 31 | 23 | - |
| #1,2,3,4,5,8,9,10&11 | 33 | 24 | - |
| Nuts, Exhaust Pipe to Catalytic Converter Flange | 27 | - | 19 |
| Nuts, Catalytic Converter to Turbocharger | 27 | 19 | - |
| Bolt, DPF to Catalytic Converter | 32 | 24 | - |
| Bolts, DPF to Muffler | 32 | 24 | - |
| Clamp, Tailpipe to Rear Tailpipe Hanger | 27 | 20 | - |
| Oxygen Sensors | 27 | 20 | - |
| Heat Shields | 45 | 33 | - |

| FRAME AND BUMPER |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs | In. Lbs. |
| Front bumper support bolts | 90 | 66.5 | — |
| Fuel tank skid plate to crossmember bolts | 99 | 73 | — |
| Rear bumper support bolts | 25 | 18.5 | — |
| Rear bumper support bracket to frame bolts | 55 | 40.5 | — |
| Trailer hitch/rear tow hook bolts | 81 | 60 | — |
| Transfer case skid plate crossmember bolts | 65 | 48 | — |
| Transfer case skid plate support bracket bolts | 65 | 48 | — |
| Transmission crossmember bolts | 122 | 90 | — |
| Transmission skid plate support crossmember bolts | 65 | 48 | — |
| Transmission skid plate to crossmember bolt | 65 | 48 | — |

| TORQUE - 3.8L GAS |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Accelerator Pedal Bracket Mounting | 12 | - | 105 |
| Crankshaft Position Sensor - 3.8L | 28 | 21 | - |
| Camshaft Position Sensor - 3.8L | - | - | 106 |
| Fuel Filler Hose Clamp at Tank | 3 | - | 30 |
| Fuel Filler Housing-to-Body Screws | 2 | - | 17 |
| Fuel Rail Mounting Bolts - 3.8L | 22 | 17 | 200 |
| Fuel Tank Mounting Straps | 41 | 30 | - |
| Throttle Body Mounting Bolts - 3.8L | 7.5 | 5.5 | 65 |
| Oxygen Sensors | 41 | 30 | - |

| STEERING COLUMN |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Steering Wheel Nut | 54 | 40 | — |
| Steering Column Mounting Nuts | 17 | — | 150 |
| Intermediate Shafts Coupler Pinch Bolts Upper & Lower | 49 | 36 | — |
| Upper Intermediate Shaft Mounting Nuts | 13 | — | 115 |

| POWER STEERING GEAR |  |
| --- | --- |
| DESCRIPTION | SPECIFICATION |
| Steering Gear Type | Recirculating Ball |
| Steering Gear Overall Ratio | 14:1 LWB or 16:1 SWB |

| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| --- | --- | --- | --- |
| Pitman Shaft Overcenter Drag | 0.45-0.80 + Worm Shaft Preload | — | 4-7 + Worm Shaft Preload |
| Power Steering Gear Adjustment Screw Locknut | 65 | 48 | — |
| Power Steering Gear to Frame Bolts | 108 | 870 | — |
| Power Steering Gear Pitman Shaft Nut | 251 | 185 | — |
| Power Steering Gear Pressure Line | 36 | 27 | — |
| Power Steering Gear Return Line | 36 | 27 | — |

| STEERING LINKAGE |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Pitman Arm Shaft | 251 | 185 | — |
| Drag Link Ball Stud to Pitman Arm | 105 | 77 | — |
| Drag Link Ball Stud to Knuckle | 85 | 63 | — |
| Drag Link Clamp | 35 | 26 | — |
| Tie Rod Ends Ball Studs | 85 | 63 | — |
| Tie Rod Ends Clamp | 61 | 45 | — |
| Steering Damper to Frame | 68 | 50 | — |
| Steering Damper Tie Rod nut | 68 | 50 | — |
| Track Bar Frame Bracket Nut | 169 | 125 | — |
| Track Bar Axle Bracket Bolt | 169 | 125 | — |

| STEERING PUMP |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Power Steering Pump to engine | 26 | 19 | — |
| Diesel Power Steering Pump Pulley | 28 | 21 | — |
| Power Steering Pump Pressure Line | 31 | 23 | — |

| MANUAL TRANSMISSION - NSG370 |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Drain Plug | 30 | 22 | - |
| Fill Plug | 30 | 22 | - |
| Shift Tower Bolts | 14 | 10 | - |
| Shift Lever Bolt | 4 | - | 33 |
| Housing Bolts | 28 | 20 | - |
| Input Shaft Retainer Bolts | 9 | - | 80 |
| Bearing Retainer Bolts | 10 | - | 88 |
| Shift Rail Support Bolts | 8 | - | 71 |
| Idler Gear Shaft Bolt | 20 | 15 | - |
| Countershaft Bolt | 100 | 74 | - |
| Transmission - Top Four Bolts | 40 | 30 | - |
| Transmission - Side Two Bolts | 68 | 50 | - |
| Transmission - Bottom Four Bolts | 54 | 40 | - |
| Crossmember Bolts | 47 | 35 | - |
| Transmission Mount Bolts | 47 | 35 | - |

| GEAR RATIOS |  |
| --- | --- |
| GEAR | RATIO |
| FIRST | 4.46 |
| SECOND | 2.61 |
| THIRD | 1.72 |
| FOURTH | 1.24 |
| FIFTH | 1 |
| SIXTH | 0.84 |
| REVERSE | 4.06 |

| 42RLE AUTOMATIC TRANSMISSION |  |
| --- | --- |
| GENERAL SPECIFICATIONS |  |
| Transmission Type | Four-Speed Automatic, Electronically Controlled, Fully Adaptive, Electronically Modulated Torque Converter |
| Lubrication Method | Pump (internal - external gear-type) |
| Cooling Method | Water Heat Exchanger / Air-to-Oil Heat Exchanger |

| GEAR RATIOS |  |
| --- | --- |
| GEAR | RATIO |
| 1st Gear | 2.84:1 |
| 2nd Gear | 1.57:1 |
| 3rd Gear (Direct) | 1.00:1 |
| 4th Gear (Overdrive) | 0.69:1 |
| Reverse Gear | 2.21:1 |

| BEARING PRELOAD (Drag Torque) |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Output Shaft | 0.22-0.903 N·m | 1-8 in. lbs. |

| CLUTCH PACK |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Low/Reverse Clutch (Select Reaction Plate) | 0.84-1.60 mm | 0.033-0.063 in. |
| Two/Four Clutch (No Select) | 0.76-2.64 mm | 0.030-0.104 in. |
| Reverse Clutch (Select Snap Ring) | 0.89-1.37 mm | 0.035-0.054 in. |
| Overdrive Clutch (No Select) | 1.07-3.25 mm | 0.042-0.128 in. |
| Underdrive Clutch (Select Reaction Plate) | 0.94-1.50 mm | 0.037-0.059 in. |

| INPUT SHAFT |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| End Play | 0.127-0.635 mm | 0.005-0.025 in. |

| OIL PUMP CLEARANCES |  |  |
| --- | --- | --- |
| DESCRIPTION | METRIC | STANDARD |
| Outer Gear-to-Crescent | 0.060-0.298 mm | 0.0023-0.0117 in. |
| Inner Gear-to-Crescent | 0.093-0.385 mm | 0.0036-0.0151 in. |
| Outer Gear-to-Pocket | 0.089-0.202 mm | 0.0035-0.0079 in. |
| Outer Gear Side Clearance | 0.020-0.046 mm | 0.0008-0.0018 in. |
| Inner Gear Side Clearance | 0.020-0.046 mm | 0.0008-0.0018 in. |

| TORQUE SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Bolt, Converter-to-Driveplate | 88 | 65 | - |
| Bolt, Fluid Filter-to-Valve Body | 5 | - | 45 |
| Bolt, L/R Clutch Retainer-to-Case | 5 | - | 45 |
| Bolt, Adapter/Extension Housing | 54 | 40 | - |
| Bolt, Manual Valve Lever-to-Manual Valve | 5 | - | 45 |
| Bolt, Oil Pan-to-Case | 20 | 14.5 | - |
| Bolt, Oil Pump-to-Case | 30 | - | 265 |
| Bolt, Park Sprag Retainer | 4.5 | - | 40 |
| Bolt, Reaction Shaft Support Halves | 28 | - | 250 |
| Bolt, Solenoid/Pressure Switch Assy-to-Valve Body | 5.5 | - | 50 |
| Bolt, Torque Converter Housing to Engine Block | 129 | 95 |  |
| Bolt, Valve Body-to-Case | 12 | - | 105 |
| Bolt, Valve Body-to-Transfer Plate | 5 | - | 45 |
| Fitting, Cooler Line | 47.5 | 35 | - |
| Nut, Output Shaft | 271 | 200 | - |
| Plug, Pressure Tap | 5 | - | 45 |
| Bolt, Input Speed-to-Case Sensor | 9 | - | 80 |
| Bolt, Output Speed-to-Case Sensor | 9 | - | 80 |

| 545RFE Transmission |  |  |
| --- | --- | --- |
| COMPONENT | METRIC | STANDARD |
| Output Shaft End Play | 0.22-0.55 mm | 0.009-0.021 in. |
| Input Shaft End Play | 0.46-0.89 mm | 0.018-0.035 in. |
| 2C Clutch Pack Clearance | 0.455-1.335 mm | 0.018-0.053 in. |
| 4C Clutch Pack Clearance | 0.770-1.390 mm | 0.030-0.055 in. |
| L/R Clutch Pack Clearance | 1.00-1.74 mm | 0.039-0.069 in. |
| OD Clutch Pack Clearance | 1.103-1.856 mm | 0.043-0.073 in. |
| UD Clutch Pack Clearance | 0.84-1.54 mm | 0.033-0.061 in. |
| Reverse Clutch Pack Clearance | 0.81-1.24 mm | 0.032-0.049 in. |
| Recommended fluid | Mopar® ATF +4 |  |

| GEAR RATIOS |  |
| --- | --- |
| 1ST | 3.00:1 |
| 2ND | 1.67:1 |
| 2ND PRIME | 1.50:1 |
| 3RD | 1.00:1 |
| 4TH | 0.75:1 |
| 5TH | 0.67:1 |
| REVERSE | 3.00:1 |

| TORQUE SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Fitting, cooler line at trans | 17.5 | - | 155 |
| Bolt, torque converter housing to engine upper four | 41 | 30 |  |
| Bolt, torque converter housing to engine lower four | 54 | 40 |  |
| Bolts, transmission collar | 68 | 50 |  |
| Bolt, torque converter to driveplate | 31 | 23 | 270 |
| Bolt/nut, crossmember | 68 | 50 | - |
| Bolt, driveplate to crankshaft | 75 | 55 | - |
| Bolts, clevis bracket / rear support | 47 | 35 |  |
| Bolt , converter housing | 68 | 50 |  |
| Bolt, oil pan | 12 | - | 105 |
| Screw, primary fluid filter | 4.5 | - | 40 |
| Nuts, transfer case | 35 | 26 |  |
| Oil Filter, Cooler Return | 9.5 |  | 84 |
| Bolt, oil pump | 28 | - | 250 |
| Bolt, oil pump body to cover | 4.5 | - | 40 |
| Screw, plate to oil pump body | 4.5 | - | 40 |
| Bolt, valve body to case | 12 | - | 105 |
| Plug, pressure test port | 5 | - | 45 |
| Bolt, reaction shaft support | 12 | - | 105 |
| Screw, valve body to transfer plate | 5.5 | - | 50 |
| Screw, solenoid module to transfer plate | 6 | - | 50 |
| Screw, accumulator cover | 7 | - | 60 |
| Screw, detent spring | 4.5 | - | 40 |
| Bolt, input speed sensor | 12 | - | 105 |
| Bolt, output speed sensor | 12 | - | 105 |
| Bolt, line pressure sensor | 12 | - | 105 |
| Bolt, extension housing | 54 | 40 | - |
| Screw, manual valve cam retaining | 4.5 | - | 40 |
| Screw, manual selector shaft retaining | 28 | - | 250 |
| Cross-bolt, manual selector shaft | 16 | - | 140 |

| TRANSFER CASE - NV241 GENII |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N-m | Ft. Lbs. | In. Lbs. |
| Plug, Detent | 16-24 | 39434 | - |
| Plug, Drain/Fill | 20-34 | 15-25 | - |
| Bolt, Extension Housing | 16-24 | 39434 | - |
| Bolt, Case Half | 20-27 | 15-24 | - |
| Screw, Oil Pump | 39432 | 39306 | - |
| Nut, Front Companion Flange | 122-176 | 90-130 | - |
| Nut, Range Lever | 27-34 | 20-25 | - |
| Sector Support | 27-42 | 20-30 | - |
| Nuts, Mounting | 30-41 | 20-30 | - |
| Position Sensor | 20-34 | 16-25 | - |

| TRANSFER CASE - NV241OR |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Plug, Detent | 16-25 | 39434 | - |
| Plug, Drain/Fill | 20-34 | 15-25 | - |
| Bolt, Rear Extension | 16-24 | 39434 | - |
| Bolt, Front Input Retainer | 21 | 16 | - |
| Bolt, Case Half | 20-27 | 15-20 | - |
| Nut, Front Yoke | 122-176 | 90-130 | - |
| Nut, Rear Companion Flange | 258-312 | 190-230 | - |
| Bolt, Shift Lever | 20-34 | 15-25 | - |
| Nuts, Mounting | 30-41 | 20-30 | - |
| Bolts, U-Joint | 19 | 17 | - |
| Support, Sector | 27-42 | 20-30 | - |
| Sensor, Transfer Case Position | 20-34 | 16-25 | - |

| WHEELS |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Lug Nut 1/2 X 20 with 60° Cone | 115-170 | 85-125 | — |
| Spare Wheel Lug Nut | 54–108 | 40–80 | — |

| TIRE PRESSURE MONITOR |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Tire Pressure Sensor/Transmitter Mounting Nut | 8 | — | 71 |

| BODY |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs | In. Lbs. |
| A/c expansion valve nut | 23 | 17 | — |
| Backlite hinge bolts | 10 | — | 90 |
| Body isolator mounting bolts | 108 | 80 | — |
| Body isolator to body nuts | 54 | 40 | — |
| Brake booster bracket nuts | 18 | 13.5 | — |
| Center OCS strain gauge stud nut | 45 | 33 | — |
| Door carrier plate screws | 10 | — | 90 |
| Door hinge pin nut | 8.5 | — | 75 |
| Door hinge to body bolts | 28 | 21 | — |
| Door hinge to hinge bolts | 26 | 19 | — |
| Door latch adjustment screw | 3 | — | 26.5 |
| Door latch mounting screws | 8 | — | 71 |
| Door latch striker bolts | 26 | 19 | — |
| Four door rear seat 40% inner seat back pivot nut | 25 | 18.5 | — |
| Four door rear seat center belt buckle bolt | 61 | 45 | — |
| Four door rear seat cushion pivot bolts | 25 | 18.5 | — |
| Four door rear seat front mounting fasteners | 61 | 45 | — |
| Four door rear seat headrest bolts | 8 | — | 71 |
| Four door rear seat inner latch floor bolts | 25 | 18.5 | — |
| Four door rear seat latch to back frame bolts | 45 | 33 | — |
| Four door rear seat outboard latch floor bolts | 25 | 18.5 | — |
| Four door rear seat rear mounting bolt | 61 | 45 | — |
| Four door rear seat rear mounting nuts | 85 | 62.5 | — |
| Four door rear seat seat cushion pivot bolt | 25 | 18.5 | — |
| Four door retractor screw | 55 | 41 | — |
| Four door seat cushion pivot bolt | 25 | 18.5 | — |
| Front door exterior handle nut | 4.5 | — | 40 |
| Front door glass run nut | 10 | — | 90 |
| Front door interior handle screw | 8.5 | — | 75 |
| Front seat back assembly bolts | 47 | 34.5 | — |
| Front seat belt anchor bolt | 40 | 29.5 | — |
| Front seat belt buckle bolt | 50 | 37 | — |
| Front seat belt turning loop screw | 40 | 29.5 | — |
| Front seat mounting bolts | 61 | 45 | — |
| Front seat riser nuts | 28 | 21 | — |
| Half door latch nut | 4.5 | — | 40 |
| Hard top front bolts | 10.5 | — | 93 |
| Hard top side bolts | 18 | 13 | — |
| Hood catch bracket nuts | 7 | — | 62 |
| Hood catch nuts | 15 | 11 | — |
| Hood hinge to body bolts | 24 | 18 | — |
| Hood hinge to hood bolts | 26 | 19 | — |
| Hood safety catch bolt | 11 | 8 | 97 |
| Instrument panel center support bracket fasteners | 8 | 6 | 71 |
| Instrument panel fence line nuts | 12 | 9 | — |
| Instrument panel side support bolts | 11 | 8 | — |
| Lower fender to sill bolts | 12 | 9 | — |
| Manual sifter bolt | 33 | 24.5 | — |
| OCS riser nuts | 28 | 21 | — |
| Outside rearview mirror bolts | 13 | 10 | — |
| Rear door exterior handle nut | 4.5 | — | 40 |
| Rear door glass run bolt | 12 | 9 | — |
| Rear door glass run nut | 10 | — | 90 |
| Rear door interior handle screw | 8.5 | — | 75 |
| Rear view mirror set screw | 2 | — | 18 |
| Seat side airbag nuts | 7 | — | 62 |
| Side step bracket bolts | 24 | 18 | — |
| Side step sill nuts | 12 | 9 | — |
| Soft top pivot knuckle bolts | 14 | 10 | — |
| Speaker pod screws | 10 | — | 90 |
| Sport bar cross bar bracket bolts | 19 | 14 | — |
| Sport bar front cross bar bracket bolts | 19 | 14 | — |
| Sport bar rear bolts | 19 | 14 | — |
| Sport bar rear cross bar bracket bolts | 19 | 14 | — |
| Sport bar side bar bolts | 19 | 14 | — |
| Sport bar side bar to windshield frame bolts | 23 | 17 | — |
| Sport bar to b-pillar bolts | 19 | 14 | — |
| Steering column opening reinforcement plate bolts | 11 | 8 | — |
| Tailgate exterior handle nuts | 4.5 | — | 40 |
| Tailgate hinge to body bolts | 26 | 19 | — |
| Tailgate hinge to tailgate bolts | 20 | 15 | — |
| Tailgate latch screws | 8 | — | 71 |
| Tailgate stabilizer insert and cup bolts | 10 | — | 90 |
| Tailgate striker bolts | 28 | 21 | — |
| Two door rear seat back frame to cushion bolts | 50 | 37 | — |
| Two door rear seat back latch bolts | 45 | 33 | — |
| Two door rear seat center belt buckle bolt | 55 | 41 | — |
| Two door rear seat cushion to back bolts | 50 | 37 | — |
| Two door rear seat front latch cushion frame bolts | 45 | 33 | — |
| Two door rear seat rear latch nuts | 10 | — | 90 |
| Upper fender fasteners | 5 | — | 44 |
| Windshield frame hinge to body bolts | 20 | 15 | — |
| Windshield frame reinforcement bracket bolts | 20 | 15 | — |

| A/C SYSTEM |  |  |
| --- | --- | --- |
| ITEM | DESCRIPTION | NOTES |
| A/C Compressor | Zexel DKS-17DS | ZXL-100 PAG Oil |
| Freeze-up Control | Evaporator Temperature Sensor | HVAC housing mounted - input to powertrain control module (PCM) or engine control module (ECM), depending on engine application |
| Low PSI Control | A/C Pressure Transducer | A/C discharge line mounted - input to powertrain control module (PCM) or engine control module (ECM), depending on engine application - PCM/ECM disengages compressor clutch below 0.451 volts |
| High PSI Control | A/C Pressure Transducer | A/C discharge line mounted - input to powertrain control module (PCM) or engine control module (ECM), depending on engine application - PCM/ECM disengages compressor clutch above 4.519 volts |
|  | High Pressure Relief Valve | A/C Compressor mounted - opens at a discharge pressure over 3430 - 3930 kPa (497 - 570 psi) |
| R-134a Refrigerant Charge Capacity | 0.51 kg (1.125 lbs). | Also see A/C Underhood Specification Label located in the engine compartment |
| A/C Clutch Coil Draw | 3.3 amps | @ 12V ± 0.5V @ 25° C (77° F) |
| A/C Clutch Coil Resistance | 4.2 ± 0.2 ohms | When measured across coil lead connector @ 25° C (77° F) |
| A/C Clutch Air Gap | 0.30 - 0.60 mm (0.012 - 0.024 in.) |  |

| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| --- | --- | --- | --- |
| All Screws NOT Listed Below | 1.2 | – | 10 |
| A/C Compressor Bolts | 28 | 21 | – |
| A/C Compressor Clutch Field Coil Screws | 5 | – | 44 |
| A/C Compressor Shaft Bolt | 20 | – | 177 |
| A/C Compressor Field Coil Connector Bracket Screw | 4.3 | – | 38 |
| A/C Condenser to Radiator Bolts | 5 | – | 45 |
| A/C Heater Control Screws | 2 | – | 17 |
| Air Inlet Housing to Instrument Panel Support Bolts | 4 | – | 35 |
| Blend Door Lever Screw | 0.6 | – | 5 |
| Discharge Line to Condenser Nut | 23 | 17 | – |
| Discharge Line to Compressor Nut | 23 | 17 | – |
| Expansion Valve Bolts | 11 | – | 97 |
| HVAC Housing to Instrument Panel Support Bolts | 4 | – | 35 |
| Liquid Line to Condenser Nut | 23 | 17 | – |
| Mode Door Cam Screw | 0.6 | – | 5 |
| Receiver/Drier to Condenser Bolt | 22 | 16 | – |
| Suction Line to Compressor Nut | 23 | 17 | – |
| Suction and Liquid Lines to Expansion Valve Nut | 8 | – | 70 |

| LAMPS |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | N·m | Ft. Lbs. | In. Lbs. |
| Center High Mounted Stop Lamp (CHMSL) Mounting Screws | 2 | - | 20 |
| Front Fog Lamp Mounting Screws | 2 | - | 18 |
| Left Multi-Function Switch Mounting Screw | 1 | - | 10 |
| License Plate Bracket Mounting Screws | 1.5 | - | 15 |
| Park Brake Switch Mounting Screw | 2.5 | - | 24 |
| Rear Fog Lamp Mounting Screws | 3 | - | 27 |
| Rear Lamp Unit Mounting Screws | 1.5 | - | 15 |